Name: Dang Hung Thinh

From: HCMUTE

Major: Computer engineering

--

Lab 05-GCD Calculator

The objectives of this project are as follows:

- Become familiar with SystemC and develop a SystemC model at the behavioral level to calculate the Greatest Common Divisor (GCD) of two integers.

- Simulate and test the designed components within the SystemC environment.

- Then, implement the design in HDL (Verilog), simulate it, and observe input/output waveforms using Xilinx simulation tools.

- Next, use Yosys to map the Verilog netlist to the FreePDK45nm standard cell library.

- Finally, import the mapped Verilog netlist into Cadence Virtuoso to generate the schematic, functional, and symbol views, and analyze the waveform, delay, and power consumption of the circuit.

Introduction

The algorithm used to compute the GCD is as follows. Two numbers are compared ( x = y ?). If so the the GCD is found. If x > y, then x = x - y. The two numbers are then compared once again. If y > x, then y = y - x. The two numbers are then compared once again. Here is and example of our algorithim:

x = 10
y = 2

Is x = y? No, x > y therefore x = x - y
in our case, x = 10 - 2 = 8.

Is x = y? No, x > y therefore x = x - y
In our case, x = 8 - 2 = 6.

Is x = y? No, x > y there fore x = x - y
In our case, x = 6 - 2 = 4.

Is x = y? No, x > y therefore x = x - y
In our case, x = 4 - 2 = 2.

Is x = y? Yes, therefore the GCD of 10 and 2 is 2.

Note that 0 is not a valid input.

Design Problems

1. Implement a finite-state machine (FSM)  in SystemC to calculate the Greatest Common Divisor (GCD) of 2 numbers.

The design of the GCD calculator should be divided into 2 parts - a controller and a datapath. The controller is an FSM which issues commands to the datapath based on the current state and the external inputs. This can be a behavioral description. The datapath contains a netlist of functional units like multiplexors, registers, subtractors and a comparator, and hence this design is structural. The controller basically steps through the GCD algorithim shown above. If x = y, we have finished computing the GCD, and we go to the final state and assert the data output line. The Datapath does the actual GCD computation. It has the following components:

- Mux: takes 2 4-bit inputs and one select line. Based on the select line, it outputs either the 1st 4-bit number or the 2nd 4-bit number.

- Register: Takes a 4-bit input, a load signal, reset, and a clock signal. If the load signal is high and the clock is pulsed, it outputs the 4-bit number.

- Comparator: Takes 2 4-bit numbers, and assets one of 3 signals depending on whether the 1st number is less than, greater than or equal to the 2nd number.

- Subtractor: Takes 2 4-bit numbers, subtracts the smaller number from the larger.

- Output Register: Holds the GCD value. When x = y the GCD has been found and can be outputted. Because it is a register entity it should also take a clock and reset signal.

Sample Structure of the Controller and Datapath

--

1. Using SystemC to implement the algorithm described above

7.2 Mux block (4 bit) - LIBRARY MUX(INPUT 4BIT): #ifndef MUX_H #define MUX_H #include <systemc.h> SC_MODULE(Mux) { sc_in<sc_uint<4>> in0, in1; sc_in<bool> sel; sc_out<sc_uint<4>> out; void do_mux() { if (sel.read() == 0) out.write(in0.read()); else out.write(in1.read()); } SC_CTOR(Mux) { SC_METHOD(do_mux); sensitive << in0 << in1 << sel; } }; #endif

7.3 Register 4 bit - LIBRARY REGISTER 4BIT: #ifndef REGISTER_H #define REGISTER_H #include <systemc.h> SC_MODULE(Register) { sc_in<sc_uint<4>> d; sc_in<bool> load, reset, clk; sc_out<sc_uint<4>> q; sc_uint<4> temp; void do_register() { if (reset.read()) { temp = 0; } else if (load.read() && clk.posedge()) { temp = d.read(); } q.write(temp); } SC_CTOR(Register) { SC_METHOD(do_register); sensitive << clk.pos(); } }; #endif

7.3. C omparator 4 bit - LIBRARY COMPARATOR 4 BIT: #ifndef COMPARATOR_H #define COMPARATOR_H #include <systemc.h> SC_MODULE(Comparator) { sc_in<sc_uint<4>> a, b; sc_out<bool> a_gt_b, a_lt_b, a_eq_b; void do_compare() { if (a.read() > b.read()) { a_gt_b.write(true); a_lt_b.write(false); a_eq_b.write(false); } else if (a.read() < b.read()) { a_gt_b.write(false); a_lt_b.write(true); a_eq_b.write(false); } else { a_gt_b.write(false); a_lt_b.write(false); a_eq_b.write(true); } } SC_CTOR(Comparator) { SC_METHOD(do_compare); sensitive << a << b; } }; #endif

7.4. Subtractor 4 bit - LIBRARY COMPARATOR 4 BIT: #ifndef SUBTRACTOR_H #define SUBTRACTOR_H #include <systemc.h> SC_MODULE(Subtractor) { sc_in<sc_uint<4>> a, b; sc_out<sc_uint<4>> out; void do_subtract() { if (a.read() > b.read()) out.write(a.read() - b.read()); else out.write(b.read() - a.read()); } SC_CTOR(Subtractor) { SC_METHOD(do_subtract); sensitive << a << b; } }; #endif

7.4. Register output 4 bit - LIBRARY REGISTER OUTPUT 4 BIT: // OutputRegister.h #include <systemc.h> SC_MODULE(OutputRegister) { sc_in<bool> clk, reset, enable; sc_in<sc_uint<4>> d_in; sc_out<sc_uint<4>> d_out; sc_uint<4> reg; void do_output() { if (reset.read()) reg = 0; else if (clk.posedge()) { if (enable.read()) reg = d_in.read(); } d_out.write(reg); } SC_CTOR(OutputRegister) { SC_METHOD(do_output); sensitive << clk.pos(); } };

7.5. Diogram FSM of controller #ifndef CONTROLLER_H #define CONTROLLER_H #include <systemc.h> SC_MODULE(Controller) { sc_in<bool> clk, reset; sc_in<bool> a_gt_b, a_lt_b, a_eq_b; sc_out<bool> sel_x, sel_y; sc_out<bool> load_x, load_y, load_gcd; sc_out<sc_uint<2>> state; enum States {IDLE = 0, CALCULATE = 1, DONE = 2}; sc_signal<States> current_state, next_state; void state_transition() { if (reset.read()) current_state = IDLE; else if (clk.posedge()) current_state = next_state; } void next_state_logic() { switch (current_state.read()) { case IDLE: next_state = CALCULATE; break; case CALCULATE: if (a_eq_b.read()) next_state = DONE; else next_state = CALCULATE; break; case DONE: next_state = IDLE; break; } state.write(current_state.read()); } void output_logic() { switch (current_state.read()) { case IDLE: sel_x.write(0); sel_y.write(0); load_x.write(1); load_y.write(1); load_gcd.write(0); break; case CALCULATE: if (a_gt_b.read()) { sel_x.write(1); sel_y.write(0); load_x.write(1); load_y.write(0); load_gcd.write(0); } else if (a_lt_b.read()) { sel_x.write(0); sel_y.write(1); load_x.write(0); load_y.write(1); load_gcd.write(0); } else { sel_x.write(0); sel_y.write(0); load_x.write(0); load_y.write(0); load_gcd.write(1); } break; case DONE: sel_x.write(0); sel_y.write(0); load_x.write(0); load_y.write(0); load_gcd.write(0); break; } } SC_CTOR(Controller) { SC_METHOD(state_transition); sensitive << clk.pos(); SC_METHOD(next_state_logic); sensitive << current_state << a_gt_b << a_lt_b << a_eq_b; SC_METHOD(output_logic); sensitive << current_state << a_gt_b << a_lt_b << a_eq_b; } }; #endif

-LIBRARY DATAPATH: #ifndef DATAPATH_H #define DATAPATH_H #include <systemc.h> #include "Mux.h" #include "Register.h" #include "Comparator.h" #include "Subtractor.h" SC_MODULE(Datapath) { sc_in<bool> clk, reset; sc_in<bool> sel_x, sel_y; sc_in<bool> load_x, load_y, load_gcd; sc_in<sc_uint<4>> x_in, y_in; sc_out<bool> a_gt_b, a_lt_b, a_eq_b; sc_out<sc_uint<4>> gcd_out; sc_signal<sc_uint<4>> x, y, mux_x_out, mux_y_out, subtract_out; Mux *mux_x, *mux_y; Register *reg_x, *reg_y, *reg_gcd; Comparator *comp; Subtractor *sub; SC_CTOR(Datapath) { mux_x = new Mux("mux_x"); mux_x->in0(x_in); mux_x->in1(subtract_out); mux_x->sel(sel_x); mux_x->out(mux_x_out); mux_y = new Mux("mux_y"); mux_y->in0(y_in); mux_y->in1(subtract_out); mux_y->sel(sel_y); mux_y->out(mux_y_out); reg_x = new Register("reg_x"); reg_x->d(mux_x_out); reg_x->load(load_x); reg_x->reset(reset); reg_x->clk(clk); reg_x->q(x); reg_y = new Register("reg_y"); reg_y->d(mux_y_out); reg_y->load(load_y); reg_y->reset(reset); reg_y->clk(clk); reg_y->q(y); sub = new Subtractor("sub"); sub->a(x); sub->b(y); sub->out(subtract_out); comp = new Comparator("comp"); comp->a(x); comp->b(y); comp->a_gt_b(a_gt_b); comp->a_lt_b(a_lt_b); comp->a_eq_b(a_eq_b); reg_gcd = new Register("reg_gcd"); reg_gcd->d(x); reg_gcd->load(load_gcd); reg_gcd->reset(reset); reg_gcd->clk(clk); reg_gcd->q(gcd_out); } }; #endif

- TOP MODULE: // Top.h #ifndef TOP_H #define TOP_H #include <systemc.h> SC_MODULE(Top) { sc_in<bool> clk, reset, go; sc_in<sc_uint<4>> x_in, y_in; sc_out<sc_uint<4>> gcd_out; sc_out<sc_uint<2>> state_out; sc_signal<sc_uint<4>> x, y; enum State { IDLE, WORK, DONE }; sc_signal<State> state; void fsm() { if (reset.read() == 1) { state.write(IDLE); gcd_out.write(0); } else { switch(state.read()) { case IDLE: if (go.read() == 1) { x.write(x_in.read()); y.write(y_in.read()); // --- Fix ở đây: check nếu x hoặc y = 0 --- if (x_in.read() == 0) { gcd_out.write(y_in.read()); state.write(DONE); } else if (y_in.read() == 0) { gcd_out.write(x_in.read()); state.write(DONE); } else { state.write(WORK); } } break; case WORK: if (x.read() > y.read()) { x.write(x.read() - y.read()); } else if (y.read() > x.read()) { y.write(y.read() - x.read()); } else { // x == y gcd_out.write(x.read()); state.write(DONE); } break; case DONE: // Stay here until reset break; } } state_out.write(state.read()); } SC_CTOR(Top) { SC_METHOD(fsm); sensitive << clk.pos(); } }; #endif // TOP_H

- TEST BENCH: #include <systemc.h> #include "Top.h" // Đảm bảo bạn có Top.h và Top.cpp đúng int sc_main(int argc, char* argv[]) { sc_signal<bool> clk, reset, go; sc_signal<sc_uint<4>> x_in, y_in; sc_signal<sc_uint<4>> gcd_out; sc_signal<sc_uint<2>> state_out; Top top("top"); // Kết nối top.clk(clk); top.reset(reset); top.go(go); top.x_in(x_in); top.y_in(y_in); top.gcd_out(gcd_out); top.state_out(state_out); // Tạo file VCD trace sc_trace_file *wf = sc_create_vcd_trace_file("gcd_waveform"); sc_trace(wf, clk, "clk"); sc_trace(wf, reset, "reset"); sc_trace(wf, go, "go"); sc_trace(wf, x_in, "x_in"); sc_trace(wf, y_in, "y_in"); sc_trace(wf, gcd_out, "gcd_out"); sc_trace(wf, state_out, "state_out"); // Danh sách các cặp test (x,y) int test_vectors[][2] = { {7, 14}, {12, 8}, {9, 6}, {15, 5}, {0, 5}, {7, 5} }; int num_tests = sizeof(test_vectors) / sizeof(test_vectors[0]); // Bắt đầu test từng cặp for (int t = 0; t < num_tests; ++t) { cout << "\n========== Test case " << t+1 << ": x = " << test_vectors[t][0] << ", y = " << test_vectors[t][1] << " ==========\n"; // Reset hệ thống reset.write(1); clk.write(0); sc_start(5, SC_NS); clk.write(1); sc_start(5, SC_NS); reset.write(0); clk.write(0); sc_start(5, SC_NS); clk.write(1); sc_start(5, SC_NS); // Set input x_in.write(test_vectors[t][0]); y_in.write(test_vectors[t][1]); // Gửi tín hiệu go = 1 trong 1 chu kỳ clock go.write(1); clk.write(0); sc_start(5, SC_NS); clk.write(1); sc_start(5, SC_NS); go.write(0); // Chạy clock liên tục đến khi state == DONE int cycle = 0; while (state_out.read() != 3) { // 3 == DONE clk.write(0); sc_start(5, SC_NS); clk.write(1); sc_start(5, SC_NS); cout << "Time: " << sc_time_stamp() << " ¦ State: " << state_out.read() << " ¦ GCD: " << gcd_out.read() << endl; cycle++; if (cycle > 50) { // tránh bị kẹt vô hạn cout << "Error: Timeout!\n"; break; } } // In kết quả cuối cùng cout << "** Result: GCD(" << test_vectors[t][0] << ", " << test_vectors[t][1] << ") = " << gcd_out.read() << "\n"; } sc_close_vcd_trace_file(wf); return 0; } - OUTPUT:

--

2. Using Verilog to describe the hardware and simulate it using Xilinx tools

2.2. Tree files

Figure 2.1. Tree files in Xilinx

2.3. Source

- Subtractor module: module subtractor ( input [7:0] a, input [7:0] b, output [7:0] result ); assign result = a - b; endmodule

- Register (save input a,b): module gcd_register ( input clk, input reset, input load, input [7:0] d, output reg [7:0] q ); always @(posedge clk or posedge reset) begin if (reset) q <= 0; else if (load) q <= d; end endmodule

- Mux module: module mux2 ( input [7:0] in0, input [7:0] in1, input sel, output [7:0] out ); assign out = sel ? in1 : in0; endmodule

- Datapath Module: smodule gcd_datapath ( input clk, input reset, input load_a, load_b, input sel_a, sel_b, input [7:0] data_in1, data_in2, output [7:0] a_out, b_out, output eq, gt ); wire [7:0] a_mux_out, b_mux_out; wire [7:0] a_reg_out, b_reg_out; wire [7:0] a_sub, b_sub; // MUX chọn giữa đầu vào ban đầu và dữ liệu sau khi trừ mux2 mux_a (.in0(data_in1), .in1(a_sub), .sel(sel_a), .out(a_mux_out)); mux2 mux_b (.in0(data_in2), .in1(b_sub), .sel(sel_b), .out(b_mux_out)); // Đăng ký giá trị a, b gcd_register reg_a (.clk(clk), .reset(reset), .load(load_a), .d(a_mux_out), .q(a_reg_out)); gcd_register reg_b (.clk(clk), .reset(reset), .load(load_b), .d(b_mux_out), .q(b_reg_out)); // Bộ trừ subtractor sub_a (.a(a_reg_out), .b(b_reg_out), .result(a_sub)); subtractor sub_b (.a(b_reg_out), .b(a_reg_out), .result(b_sub)); // Comparator assign eq = (a_reg_out == b_reg_out); assign gt = (a_reg_out > b_reg_out); assign a_out = a_reg_out; assign b_out = b_reg_out; endmodule

- Module Controller (FSM): module gcd_controller ( input clk, input reset, input start, input eq, gt, output reg load_a, load_b, output reg sel_a, sel_b, output reg done ); // Các trạng thái parameter IDLE = 2'b00, LOAD = 2'b01, CALC = 2'b10, DONE = 2'b11; reg [1:0] state, next; // Bộ logic trạng thái kế tiếp + tín hiệu điều khiển always @(*) begin // Mặc định load_a = 0; load_b = 0; sel_a = 0; sel_b = 0; done = 0; case (state) IDLE: begin if (start) next = LOAD; else next = IDLE; end LOAD: begin load_a = 1; load_b = 1; next = CALC; end CALC: begin if (eq) begin next = DONE; end else if (gt) begin sel_a = 1; load_a = 1; next = CALC; end else begin sel_b = 1; load_b = 1; next = CALC; end end DONE: begin done = 1; next = IDLE; end default: next = IDLE; endcase end // Cập nhật trạng thái always @(posedge clk or posedge reset) begin if (reset) state <= IDLE; else state <= next; end endmodule

- Schematic symbol:

- GCD(28,16)=4

- GCD(0,4) =4

TCL, rtl yosys ubuntu: - Schematic in cadence:

- GCD(28,16) - Input: - Output:

- GCD(56,8) - Input: - Output: